VINS-Mono UAV

Complete Deployment Runbook

OV9281 Mono Camera  +  Pixhawk Cube  +  Raspberry Pi 5  +  Docker

ArduCopter V4.6.3  •  ROS Noetic  •  VINS-Mono VIO

SCRIPTS INCLUDED IN THIS PACKAGE

| Script | Run Where | Purpose |
| --- | --- | --- |
| ov9281_calibration.py | Container | Interactive camera intrinsics calibration with checkerboard |
| imu_camera_sync.py | Container | IMU-Camera time offset analysis + Kalibr bag recorder |
| vins_monitor.py | Container | Live terminal dashboard + pre-flight checklist |
| camera_node.py | Container | Publishes /camera/image_raw from /dev/shm/cam.mjpeg |
| vins_to_mavros.py | Container | Relays VINS odometry to /mavros/vision_pose/pose |
| takeoff_hover.py | Container | Autonomous GUIDED takeoff + hover + land |
| gcs_dashboard.py | Container | Full web GCS at http://PI_IP:5000 |
| rviz_connect.py | Laptop | Connects laptop RViz to Pi5 ROS master |
| uav_startup.sh | HOST Pi5 | One-command full stack deployment |
| uav_shutdown.sh | HOST Pi5 | Safe land + disarm + shutdown all nodes |

ISRO UAV Project  •  Confidential Operations Document

# 1. Hardware Setup — OV9281 + Pixhawk Cube

The OV9281 is a global shutter monochrome camera specifically suited for VIO because it eliminates motion blur at high frame rates. It connects to the Raspberry Pi 5 via CSI ribbon cable and is accessed through the libcamera/PiSP pipeline.

## 1.1 OV9281 Camera Specifications

| Parameter | Value | Description |
| --- | --- | --- |
| Sensor | OV9281 | Global shutter monochrome CMOS |
| Resolution | 1280x800 | Working resolution: 640x480 for VIO |
| Frame rate | 120 fps max | Use 30 fps for VINS-Mono |
| Interface | CSI-2 MIPI | Connected to Pi5 CSI ribbon port |
| Pipeline | PiSP / libcamera | Accessed via rpicam-vid on HOST |
| Shutter | Global | No motion blur — critical for VIO |
| Mono | Yes | Monochrome — VINS works on grayscale |

## 1.2 Pixhawk Cube Connection

| Parameter | Value | Description |
| --- | --- | --- |
| FCU | Pixhawk Cube | ArduCopter V4.6.3 |
| USB port | /dev/ttyACM0 | Connect via USB-C to Pi5 |
| Baud rate | 921600 | MAVROS fcu_url parameter |
| Launch file | apm.launch | Use apm.launch NOT px4.launch |
| IMU | /mavros/imu/data | 200 Hz from Pixhawk to ROS |

⛔  CRITICAL: Always use apm.launch with MAVROS. px4.launch is for PX4 firmware only. Your FCU runs ArduCopter.

# 2. OV9281 Camera Calibration

Camera calibration computes the intrinsic parameters (fx, fy, cx, cy, distortion coefficients) required by VINS-Mono to undistort images and triangulate feature points accurately. Poor calibration = drift and instability.

## 2.1 What you need

- Print a 9x6 checkerboard with 25mm squares on A4 paper (search: 'opencv checkerboard 9x6 25mm PDF')

- Mount the printed board on a rigid flat surface (cardboard or foam board)

- Good lighting — avoid glare on the checkerboard

- Camera node running (/camera/image_raw must be publishing)

## 2.2 Run calibration

# Inside container — camera node must already be running

python3 /catkin_ws/ov9281_calibration.py

The calibration tool opens a live window. Follow these steps:

- Hold checkerboard in front of camera — green corners appear when detected

- Press SPACE to capture frame — try different angles, tilts, distances

- Cover: flat center, tilted left/right, tilted up/down, corners of frame

- Capture 40 frames for best results (20 minimum)

- Press q when done — intrinsics computed automatically

- Results saved to /catkin_ws/ov9281_calibration.yaml

## 2.3 Interpreting results

| Parameter | Value | Description |
| --- | --- | --- |
| Error < 0.3 px | Excellent | Use these values directly in VINS config |
| Error 0.3–0.5 px | Good | Acceptable for stable VIO flight |
| Error 0.5–1.0 px | Marginal | Recalibrate with more diverse frames |
| Error > 1.0 px | Poor | Recalibrate — will cause VIO drift |

⚠  Recalibrate any time you physically move or remount the camera. Intrinsics change with camera position.

## 2.4 Copy results to VINS config

# The script prints the YAML block — copy it into your VINS config:

nano /catkin_ws/src/VINS-Mono/config/your_config.yaml

# Replace projection_parameters and distortion_parameters sections

# 3. IMU-Camera Synchronization

VINS-Mono requires accurate time synchronization between camera frames and IMU samples. Even a few milliseconds of offset causes significant drift. This section covers measuring and compensating for that offset.

## 3.1 Check sync quality (monitoring mode)

# Inside container

python3 /catkin_ws/imu_camera_sync.py

This monitors the time offset between IMU stamps and wall clock in real time. Look for:

- Mean offset: note this value (e.g. +12.3 ms)

- Std deviation < 2ms: good sync

- Std deviation > 5ms: hardware sync issue

## 3.2 Record Kalibr calibration bag

# Record 60 seconds of synchronized IMU + camera data

python3 /catkin_ws/imu_camera_sync.py --record --duration 60

During recording, move the drone in all 6 degrees of freedom:

- Roll left/right, pitch forward/back, yaw left/right

- Translate forward/back, left/right, up/down

- Figure-8 patterns work well

- Avoid slow drift — need dynamic motion for IMU excitation

## 3.3 Set td parameter in VINS config

# In your VINS config YAML, set the measured time offset:

td: 0.012   # camera-imu time delay in seconds (example: 12ms)

⚠  The td parameter can also be estimated online by VINS-Mono itself if you set estimate_td: 1 in the config. Recommended for first flights.

# 4. ArduCopter Parameters — Pixhawk Cube

Set these in Mission Planner (Config → Full Parameter List) or via QGroundControl. Reboot Pixhawk after changes.

## 4.1 EKF3 Vision Fusion

| Parameter | Value | Description |
| --- | --- | --- |
| AHRS_EKF_TYPE | 3 | Use EKF3 — required for visual odometry |
| EK3_ENABLE | 1 | Enable EKF3 |
| EK3_SRC1_POSXY | 6 | XY position source: ExternalNav (VIO) |
| EK3_SRC1_VELXY | 6 | XY velocity source: ExternalNav (VIO) |
| EK3_SRC1_POSZ | 1 | Z source: Barometer (safer for indoor) |
| EK3_SRC1_VELZ | 0 | Z velocity: None |
| EK3_SRC1_YAW | 6 | Yaw source: ExternalNav (VIO) |

## 4.2 Visual Odometry

| Parameter | Value | Description |
| --- | --- | --- |
| VISO_TYPE | 1 | Enable visual odometry (1=VIO type) |
| VISO_DELAY_MS | 75 | Vision pipeline delay ms — tune if oscillation |
| VISO_ORIENT | 0 | Camera orientation (0=forward-facing) |

## 4.3 GPS and Flight

| Parameter | Value | Description |
| --- | --- | --- |
| GPS_TYPE | 0 | Disable GPS for indoor / VIO-only flight |
| ARMING_CHECK | 0 | Disable arming checks (re-enable after tuning) |
| PILOT_SPEED_UP | 100 | Max climb speed cm/s |
| PILOT_SPEED_DN | 100 | Max descent speed cm/s |
| LAND_SPEED | 50 | Landing descent speed cm/s |

⛔  If drone oscillates after takeoff: increase VISO_DELAY_MS by 10ms increments until stable.

# 5. Scripts Reference — Save All to /catkin_ws/

## 5.1 camera_node.py

Reads MJPEG stream from shared memory and publishes /camera/image_raw at 30 Hz.

cat > /catkin_ws/camera_node.py << 'EOF'

import cv2, numpy as np, rospy

from sensor_msgs.msg import Image

from cv_bridge import CvBridge

rospy.init_node('camera_opencv')

pub = rospy.Publisher('/camera/image_raw', Image, queue_size=1)

bridge = CvBridge()

buf = b''

f = open('/dev/shm/cam.mjpeg', 'rb')

while not rospy.is_shutdown():

buf += f.read(4096)

s, e = buf.find(b'\xff\xd8'), buf.find(b'\xff\xd9')

if s != -1 and e != -1 and e > s:

fr = cv2.imdecode(np.frombuffer(buf[s:e+2], np.uint8), 1)

buf = buf[e+2:]

if fr is not None:

m = bridge.cv2_to_imgmsg(fr, 'bgr8')

m.header.stamp = rospy.Time.now()

pub.publish(m)

EOF

## 5.2 vins_to_mavros.py

Converts VINS odometry to PoseStamped and sends to ArduCopter EKF3.

cat > /catkin_ws/vins_to_mavros.py << 'EOF'

import rospy

from nav_msgs.msg import Odometry

from geometry_msgs.msg import PoseStamped

rospy.init_node('vins_to_mavros')

pub = rospy.Publisher('/mavros/vision_pose/pose', PoseStamped, queue_size=10)

def cb(msg):

ps = PoseStamped()

ps.header = msg.header

ps.header.frame_id = 'map'

ps.pose = msg.pose.pose

pub.publish(ps)

rospy.Subscriber('/vins_estimator/odometry', Odometry, cb)

rospy.spin()

EOF

## 5.3 takeoff_hover.py

GUIDED mode autonomous takeoff to configurable altitude, hover 30 seconds, then land.

cat > /catkin_ws/takeoff_hover.py << 'EOF'

import rospy

from geometry_msgs.msg import PoseStamped

from mavros_msgs.msg import State

from mavros_msgs.srv import CommandBool, SetMode

current_state = State()

def state_cb(msg): global current_state; current_state = msg

rospy.init_node('takeoff_hover')

rospy.Subscriber('/mavros/state', State, state_cb)

pub = rospy.Publisher('/mavros/setpoint_position/local', PoseStamped, queue_size=10)

rospy.wait_for_service('/mavros/cmd/arming')

rospy.wait_for_service('/mavros/set_mode')

arm_svc  = rospy.ServiceProxy('/mavros/cmd/arming', CommandBool)

mode_svc = rospy.ServiceProxy('/mavros/set_mode', SetMode)

rate = rospy.Rate(20)

pose = PoseStamped()

pose.pose.position.x = 0

pose.pose.position.y = 0

pose.pose.position.z = 1.0   # hover height meters

for _ in range(100): pub.publish(pose); rate.sleep()

rospy.loginfo('Setting GUIDED mode...')

while not rospy.is_shutdown():

if current_state.mode != 'GUIDED':

mode_svc(base_mode=0, custom_mode='GUIDED'); rospy.sleep(2)

else: rospy.loginfo('GUIDED active'); break

rate.sleep()

rospy.loginfo('Arming...')

while not rospy.is_shutdown():

if not current_state.armed:

arm_svc(True); rospy.sleep(2)

else: rospy.loginfo('Armed!'); break

rate.sleep()

rospy.loginfo('Hovering for 30 seconds at 1.0m...')

start = rospy.Time.now()

while not rospy.is_shutdown():

if (rospy.Time.now()-start).to_sec() > 30:

mode_svc(base_mode=0, custom_mode='LAND')

rospy.loginfo('Landing...'); break

pose.header.stamp = rospy.Time.now()

pub.publish(pose); rate.sleep()

EOF

# 6. GCS Dashboard

Real-time web interface accessible from any browser on your network. Shows all telemetry from both VINS-Mono and Pixhawk and provides full flight control.

## 6.1 Install and run

pip3 install flask flask-socketio

# Copy gcs_dashboard.py to /catkin_ws/ then:

python3 /catkin_ws/gcs_dashboard.py

✓  Open from any device on network: http://10.46.61.82:5000

## 6.2 Dashboard features

- VIO position (X/Y/Z m) and velocity (Vx/Vy/Vz m/s) from VINS-Mono odometry

- Artificial horizon with live roll/pitch visualization

- Pixhawk telemetry: altitude, heading, throttle %, groundspeed

- Raw IMU: accelerometer (m/s²) and gyroscope (rad/s) from Pixhawk

- Node health Hz bars — camera/IMU/VINS/vision_pose with color-coded status

- Status pills: FCU connected, Armed/Disarmed, Flight mode, VINS active

- Flight mode buttons: STABILIZE, GUIDED, LOITER, ALT HOLD, POS HOLD, AUTO

- ARM / DISARM / TAKEOFF (configurable altitude) / LAND / RTL buttons

- FCU status log with timestamps

# 7. Laptop RViz Visualization

Connects your laptop's RViz directly to the Pi5 ROS master over WiFi to see the full 3D VINS-Mono visualization.

## 7.1 Laptop prerequisites

sudo apt install ros-noetic-rviz ros-noetic-mavros

## 7.2 Connect and launch

# Copy rviz_connect.py to your laptop, then:

python3 rviz_connect.py --pi-ip 10.46.61.82

## 7.3 Manual method

export ROS_MASTER_URI=http://10.46.61.82:11311

export ROS_IP=$(ip route get 10.46.61.82 | awk '{print $7;exit}')

rviz

## 7.4 Key RViz displays to add

- Odometry → /vins_estimator/odometry (green arrows — live VIO pose)

- Path → /vins_estimator/path (trajectory history)

- PointCloud2 → /vins_estimator/point_cloud (tracked features)

- Image → /vins_estimator/image_track (feature tracking view)

- TF → shows all coordinate frames

- PoseStamped → /mavros/local_position/pose (EKF3 output, amber)

⚠  Laptop and Pi5 must be on the same WiFi. Fixed Frame in RViz should be set to 'world'.

# 8. System Health Monitor

vins_monitor.py provides a live terminal dashboard and pre-flight checklist validation. Run it alongside your flight operations.

## 8.1 Live dashboard mode

# Inside container

python3 /catkin_ws/vins_monitor.py

Shows in real time:

- All 5 node Hz bars with color-coded health (green/yellow/red)

- FCU armed/mode/connected state

- Live VIO X/Y/Z position

- Event log with timestamps

- FLIGHT READY / NOT FLIGHT READY overall status

## 8.2 Pre-flight checklist mode

python3 /catkin_ws/vins_monitor.py --preflight

Runs structured checklist and exits with pass/fail. Checks:

- Camera >25 Hz, IMU >90 Hz, VINS >5 Hz, Vision pose >5 Hz

- FCU connected and not armed

- VIO position values are sane (not NaN/inf/huge)

- Z-axis reasonable (0–10m range)

# 9. Complete Launch Sequence

## Option A: One-command auto-deploy (recommended)

Run this single script on HOST Pi5 — launches everything automatically:

bash /home/pi/uav_startup.sh

✓  Script handles: camera stream, Docker start, roscore, camera node, MAVROS, VINS, pose relay, GCS dashboard, monitor, pre-flight check.

## Option B: Manual step-by-step

⚠  Follow exactly this order. Never skip steps or change sequence.

# Run container : 
docker run -it   --name vins_mono   --net=host   --privileged     --device /dev/video0   --device /dev/video1   --device /dev/media0   --device /dev/media1   --device /dev/ttyACM0   --device /dev/ttyUSB0     --volume /run/udev:/run/udev:ro   --volume /run/libcamera:/run/libcamera:ro   --volume /dev/shm:/dev/shm   --volume /usr/lib/aarch64-linux-gnu/libcamera.so.0.7.0:/usr/lib/aarch64-linux-gnu/libcamera.so.0.7.0:ro   --volume /usr/lib/aarch64-linux-gnu/libcamera.so.0.7:/usr/lib/aarch64-linux-gnu/libcamera.so.0.7:ro   --volume /usr/lib/aarch64-linux-gnu/libcamera.so:/usr/lib/aarch64-linux-gnu/libcamera.so:ro   --volume /usr/lib/aarch64-linux-gnu/libcamera-base.so.0.7.0:/usr/lib/aarch64-linux-gnu/libcamera-base.so.0.7.0:ro   --volume /usr/lib/aarch64-linux-gnu/libcamera-base.so.0.7:/usr/lib/aarch64-linux-gnu/libcamera-base.so.0.7:ro   --volume /usr/lib/aarch64-linux-gnu/libcamera-base.so:/usr/lib/aarch64-linux-gnu/libcamera-base.so:ro   --volume /usr/lib/aarch64-linux-gnu/libcamera:/usr/lib/aarch64-linux-gnu/libcamera:ro   --volume /usr/share/libcamera:/usr/share/libcamera:ro   --volume /usr/share/rpicam-apps:/usr/share/rpicam-apps:ro     --volume /usr/bin/rpicam-vid:/usr/bin/rpicam-vid:ro   --volume /usr/lib/aarch64-linux-gnu/librpicam_app.so.1.11.1:/usr/lib/aarch64-linux-gnu/librpicam_app.so.1.11.1:ro   --volume /usr/lib/aarch64-linux-gnu/librpicam_app.so.1:/usr/lib/aarch64-linux-gnu/librpicam_app.so.1:ro   --volume /usr/lib/aarch64-linux-gnu/librpicam_app.so:/usr/lib/aarch64-linux-gnu/librpicam_app.so:ro     -e ROS_MASTER_URI="http://${DRONE_IP}:11311"   -e ROS_IP="${DRONE_IP}"   -e ROS_HOSTNAME="${DRONE_IP}"     isro26:latest bash

| T1 | HOST Pi5 | Before running always do : pkill -f rpicam ¦¦ true rpicam-vid --width 640 --height 480 --framerate 30 --codec mjpeg --timeout 0 --output /dev/shm/cam.mjpeg --nopreview & ps aux ¦ grep rpicam killall rpicam-vid |
| --- | --- | --- |

✓  Verify T1: watch -n1 'ls -lh /dev/shm/cam.mjpeg' — file size must grow

| T2 | Container | export ROS_MASTER_URI=http://10.46.61.82:11311 export ROS_HOSTNAME=10.46.61.82 source /opt/ros/noetic/setup.bash && roscore |
| --- | --- | --- |

| T3 | Container | export ROS_MASTER_URI=http://10.46.61.82:11311 export ROS_HOSTNAME=10.46.61.82 python3 /catkin_ws/camera_node.py |
| --- | --- | --- |

✓  Verify T3: rostopic hz /camera/image_raw → expect ~30 Hz

| T4 | Container | export ROS_MASTER_URI=http://10.46.61.82:11311 export ROS_HOSTNAME=10.46.61.82 roslaunch mavros apm.launch fcu_url:=/dev/ttyACM0:921600 |
| --- | --- | --- |

✓  Verify T4: look for 'CON: Got HEARTBEAT, connected. FCU: ArduPilot'

| T5 | Container | export ROS_MASTER_URI=http://10.46.61.82:11311 export ROS_HOSTNAME=10.46.61.82 roslaunch vins_estimator euroc.launch |
| --- | --- | --- |

⚠  Move drone slowly in front of textured surface — VINS needs 10-20s to initialize

✓  Verify T5: look for 'system is initialized' in VINS output

| T6 | Container | export ROS_MASTER_URI=http://10.46.61.82:11311 export ROS_HOSTNAME=10.46.61.82 python3 /catkin_ws/vins_to_mavros.py |
| --- | --- | --- |

| T7 | Container | export ROS_MASTER_URI=http://10.46.61.82:11311 export ROS_HOSTNAME=10.46.61.82 python3 /catkin_ws/gcs_dashboard.py |
| --- | --- | --- |

✓  T7: Open http://10.46.61.82:5000 — all nodes should show green

| T8 | Container | export ROS_MASTER_URI=http://10.46.61.82:11311 export ROS_HOSTNAME=10.46.61.82 python3 /catkin_ws/vins_monitor.py |
| --- | --- | --- |

| T9 | Laptop | python3 rviz_connect.py --pi-ip 10.46.61.82 |
| --- | --- | --- |

# 10. Pre-Flight Topic Verification

| Topic | Target Hz | Verify Command |
| --- | --- | --- |
| /camera/image_raw | 30 Hz | rostopic hz /camera/image_raw |
| /mavros/imu/data | 100-200 Hz | rostopic hz /mavros/imu/data |
| /vins_estimator/odometry | 10-30 Hz | rostopic hz /vins_estimator/odometry |
| /mavros/vision_pose/pose | 10-30 Hz | rostopic hz /mavros/vision_pose/pose |
| /mavros/state | 1 Hz | rostopic echo /mavros/state ¦ grep mode |
| /mavros/local_position/pose | 10 Hz | rostopic hz /mavros/local_position/pose |

⛔  DO NOT ARM until ALL topics are at expected Hz. A missing topic means ArduCopter EKF3 is not receiving position data and the drone will fly uncontrolled.

# 11. UAV Deployment Flight Checklist

## Pre-power checks

- Propellers tight and balanced — check each arm

- Battery fully charged — check voltage per cell

- OV9281 camera ribbon cable firmly seated

- Pixhawk USB cable to Pi5 secure and strain-relieved

- Pi5 powered via separate BEC — not USB from Pixhawk

- Frame arms all tight — no wobble

## Software startup

- SSH into Pi5 or connect keyboard/monitor

- Run: bash uav_startup.sh — wait for DEPLOYMENT COMPLETE

- Open GCS dashboard: http://PI_IP:5000

- Run pre-flight: python3 /catkin_ws/vins_monitor.py --preflight

- Verify all 6 Hz checks pass

## VIO initialization

- Pick up drone and walk around — camera must see textured surface

- Watch VIO position update on GCS dashboard

- Confirm VIO Z value is near 0.0 when drone on ground

- In Mission Planner confirm EK3 status shows ExternalNav active

## Takeoff

- Clear flight area — min 3x3 meter open space

- Set mode STABILIZE — confirm on GCS

- Arm drone — confirm ARM on GCS dashboard

- Switch to GUIDED mode on GCS or RC

- Run: python3 /catkin_ws/takeoff_hover.py

- Watch drone lift to 1.0m and hold position

- Monitor VIO position — should hold within +/- 10cm

## Emergency procedures

- RC OVERRIDE: Switch to STABILIZE on RC transmitter immediately

- LAND command: Click LAND on GCS dashboard or run uav_shutdown.sh

- DISARM immediately: Click DISARM on GCS if drone behaves erratically

- If VINS loses track: switch STABILIZE, land manually

# 12. Troubleshooting

## Camera not streaming on HOST

- Check OV9281 ribbon cable connected to Pi5 CSI port

- Run on HOST: rpicam-hello --list-cameras

- Kill and restart: sudo pkill rpicam-vid && bash uav_startup.sh

- Check dmesg for CSI/camera errors: dmesg | grep -i camera

## MAVROS not connecting to FCU

- Check USB cable: ls /dev/ttyACM0

- Fix permissions: sudo chmod 666 /dev/ttyACM0

- Confirm using apm.launch not px4.launch

- Check ArduCopter baud rate matches: 921600

## VINS-Mono not initializing

- Move drone slowly — VINS needs motion to initialize

- Point camera at textured surface (not blank wall or sky)

- Check /camera/image_raw is 30 Hz and /mavros/imu/data is 100+ Hz

- Check camera intrinsics — recalibrate if reprojection error >1px

- Check VINS config yaml has correct image_topic and imu_topic

## Drone oscillates or drifts after takeoff

- Increase VISO_DELAY_MS by 10ms (try 75 → 85 → 95)

- Recalibrate camera intrinsics with ov9281_calibration.py

- Ensure camera is rigidly mounted — no vibration

- Add vibration damping between camera and frame

- Check IMU noise params in VINS config match actual IMU

## GCS dashboard not loading

- pip3 install flask flask-socketio inside container

- Check docker run has --net=host flag

- Check port 5000: sudo netstat -tlnp | grep 5000

## RViz cannot connect to Pi5

- Both devices on same WiFi network

- Ping test: ping 10.46.61.82 from laptop

- Allow port: sudo ufw allow 11311 on Pi5

- Check ROS_IP set to laptop IP not Pi5 IP

ISRO VINS-Mono UAV Project  •  OV9281 + Pixhawk Cube  •  Complete Deployment Runbook  •  ArduCopter V4.6.3 / ROS Noetic / Pi5